DREAMFLY MOSAIC BMS

CAN Bus Protocol Reference

Packet & Message Details — All Protocols

STM32G474RET6  |  Firmware V5 + CANopen Patch Pack  |  Rev 1.0

1.  Protocol Overview

The Mosaic BMS firmware supports four CAN operating modes, selectable via GUI or CLI command 0x60. The active protocol is persisted in CalibStore. On power-up the firmware loads the stored selection and initialises the appropriate stack over FDCAN1 (STM32G474, Cortex-M4).

| Mode Index | Protocol | Standard | Default Bitrate | ID Format | Stack |
| --- | --- | --- | --- | --- | --- |
| 0 | DroneCAN | UAVCAN v0 | 1 Mbit/s | 29-bit Ext | libcanard |
| 1 | CANopen | CiA 301/302 | Selectable | 11-bit Std | CANopen_BMS (built-in) |
| 2 | CAN 2.0B Extended | J1939-style | Selectable | 29-bit Ext | Raw HAL FDCAN |
| 3 | AutoSelect | DroneCAN→2.0B | 1 Mbit/s | 29-bit Ext | FSM (probe then lock) |

2.  DroneCAN  (UAVCAN v0)

DroneCAN is the default protocol for drone/UAV flight-controller integration (Pixhawk, Cube Orange+, ArduPilot, PX4). It uses the libcanard library with 29-bit extended IDs encoded per the UAVCAN v0 specification. Dynamic Node Allocation (DNA) is supported; a static node ID can be set via the GUI.

2.1  Physical Layer

| Parameter | Value |
| --- | --- |
| Peripheral | FDCAN1 (STM32G474) |
| Mode | FDCAN_CLASSIC_CAN (FD silicon, CAN-classic frame) |
| Bit Rate | 1 Mbit/s (NominalPrescaler=34, Seg1=3, Seg2=1 @ 170 MHz) |
| ID Format | FDCAN_EXTENDED_ID — 29-bit identifiers |
| BRS / FD | Disabled (BRS_OFF, FDFormat=CLASSIC) |
| Termination | 120 Ω external — STB_CAN_Pin (PC9) de-asserted to enable driver |
| RX FIFO | FIFO0 → ISR deferred queue → main loop dispatch |
| TX | TX FIFO queue → Process_CAN_Tx() drains in main loop |

2.2  UAVCAN v0 Frame Format

Every UAVCAN frame is a standard CAN 2.0B extended frame. The 29-bit identifier encodes priority, data-type ID, and node addressing. The 8-byte payload carries a UAVCAN transfer fragment with a tail byte.

CAN Identifier — 29 bits (MSB → LSB):

| Priority [28:26] | Reserved [25:24] | Data Type ID [23:8] | Src Node ID [7:0] |
| --- | --- | --- | --- |

CAN Payload — up to 8 bytes:

| Payload[0] | Payload[1] | Payload[2] | Payload[3] | Payload[4] | Payload[5] | Payload[6] | Tail Byte |
| --- | --- | --- | --- | --- | --- | --- | --- |

Tail Byte encoding: bit 7 = Start-of-Transfer, bit 6 = End-of-Transfer, bit 5 = Toggle, bits 4:0 = Transfer ID (0–31).

2.3  Published Messages (BMS → Flight Controller)

uavcan.equipment.power.BatteryInfo  (DTID 1092 / 0x444)  — 1 Hz

| Byte(s) | Field | Type | Unit | Source in Firmware |
| --- | --- | --- | --- | --- |
| 0–3 | voltage | f32 LE | V | SOC_GetPackVoltage() |
| 4–7 | current | f32 LE | A | SOC_GetCurrent() (−=discharge) |
| 8–11 | temperature | f32 LE | K | temp_celsius + 273.15 |
| 12–15 | full_charge_capacity_wh | f32 LE | Wh | settings.battery_capacity_ah × pack_voltage |
| 16–19 | remaining_capacity_wh | f32 LE | Wh | SOC_GetRemainingAh() × pack_voltage |
| 20 | state_of_charge_pct | u8 | % | SOC_GetPercent() (0–100) |
| 21 | state_of_health_pct | u8 | % | SOH_GetLifeData().soh_percent |
| 22 | cycle_count | u16 LE | cycles | SOH_GetLifeData().cycles |
| 24 | status_flags | u8 | — | Fault_GetBitmask() & 0xFF |
| 25 | battery_id | u8 | — | settings.battery_index |
| 26–27 | model_instance_id | u16 LE | — | 0x0000 (reserved) |
| 28 | Tail Byte | u8 | — | Transfer ID ¦ SoT ¦ EoT ¦ Toggle |

uavcan.protocol.NodeStatus  (DTID 341 / 0x155)  — 1 Hz

| Byte(s) | Field | Type | Value / Source |
| --- | --- | --- | --- |
| 0–3 | uptime_sec | u32 LE | HAL_GetTick() / 1000 |
| 4 | health | u2 | 0=OK, 1=WARNING (any fault), 2=ERROR (severe) |
| 4 | mode | u3 | 0=OPERATIONAL always |
| 4–5 | sub_mode | u3 | 0 (reserved) |
| 5–6 | vendor_specific_status_code | u16 LE | Fault_GetBitmask() — 13-bit fault flags |
| 7 | Tail Byte | u8 | Transfer ID ¦ SoT ¦ EoT ¦ Toggle |

2.4  DNA Allocation Frames

When can_node == 0 (settings), the firmware enters Dynamic Node Allocation. It broadcasts allocation requests on DTID 1 and waits for an allocator (flight controller) to assign a node ID. Once allocated, canardSetLocalNodeID() is called and normal operation begins.

| Frame Direction | DTID | CAN ID Pattern | Description |
| --- | --- | --- | --- |
| BMS → Allocator | 1 | 0x0FD5500 + SrcID | Allocation request — carries preferred node ID + unique ID bytes |
| Allocator → BMS | 1 | 0x0FD5500 + AllocID | Allocation response — payload contains assigned node ID in byte 0 |

3.  CANopen  (CiA 301)

CANopen uses 11-bit standard CAN IDs structured as Function Code (4 bits) + Node ID (7 bits). The BMS acts as a CANopen device node. A self-contained NMT + Heartbeat + TPDO + SDO server is implemented in CANopen_BMS.c without requiring an external library.

3.1  Physical Layer

| Parameter | Value |
| --- | --- |
| Peripheral | FDCAN1 (FDCAN_CLASSIC_CAN) |
| ID Format | FDCAN_STANDARD_ID — 11-bit identifiers (COB-ID) |
| Default Bitrate | 1 Mbit/s (GUI-selectable: 250k / 500k / 1M) |
| Node ID | 1–127, stored in settings.canopen_node_id (default: 1) |
| NMT Boot | INITIALISING → PRE-OPERATIONAL (100 ms) → OPERATIONAL (600 ms) |
| Heartbeat | 1000 ms (default), COB-ID = 0x700 + NodeID |

3.2  COB-ID Map

| COB-ID (hex) | Function | Direction | Description |
| --- | --- | --- | --- |
| 0x000 | NMT Command | Master → BMS | Network Management (start/stop/reset) |
| 0x080 + NodeID | EMCY | BMS → Bus | Emergency object (fault events) |
| 0x180 + NodeID | TPDO1 | BMS → Bus | Pack voltage + Pack current |
| 0x280 + NodeID | TPDO2 | BMS → Bus | SOC% + SOH% |
| 0x380 + NodeID | TPDO3 | BMS → Bus | NTC1 temperature + NTC2 temperature |
| 0x480 + NodeID | TPDO4 | BMS → Bus | Fault mask + ATC status + Remaining Wh |
| 0x580 + NodeID | SDO Response | BMS → Master | SDO server reply |
| 0x600 + NodeID | SDO Request | Master → BMS | SDO expedited download (write setpoints) |
| 0x700 + NodeID | Heartbeat | BMS → Bus | NMT state byte at 1 Hz |

3.3  CANopen Frame Format

CAN Standard Identifier — 11 bits:

| Func Code [10:7] | Node ID [6:0] |
| --- | --- |

3.4  TPDO Definitions  (Broadcast 1 Hz in OPERATIONAL state)

TPDO1  —  COB-ID 0x180 + NodeID

| Byte(s) | OD Signal | Type | Unit | Source |
| --- | --- | --- | --- | --- |
| 0–3 | pack_voltage_v | FLOAT32 LE | V | SOC_GetPackVoltage() |
| 4–7 | pack_current_a | FLOAT32 LE | A | SOC_GetCurrent() (negative = discharge) |

TPDO2  —  COB-ID 0x280 + NodeID

| Byte(s) | OD Signal | Type | Unit | Source |
| --- | --- | --- | --- | --- |
| 0–3 | soc_percent | FLOAT32 LE | % | SOC_GetPercent() |
| 4–7 | soh_percent | FLOAT32 LE | % | SOH_GetLifeData().soh_percent |

TPDO3  —  COB-ID 0x380 + NodeID

| Byte(s) | OD Signal | Type | Unit | Source |
| --- | --- | --- | --- | --- |
| 0–3 | temp_ntc1_c | FLOAT32 LE | °C | temp_celsius (NTC1) |
| 4–7 | temp_ntc2_c | FLOAT32 LE | °C | temp2_celsius (NTC2) |

TPDO4  —  COB-ID 0x480 + NodeID

| Byte(s) | OD Signal | Type | Unit | Source |
| --- | --- | --- | --- | --- |
| 0–1 | fault_mask | UINT16 LE | — | Fault_GetBitmask() — 13-bit flags (see §5) |
| 2 | atc_status | UINT8 | — | ATC_GetStatus() 0=off, 1=heating |
| 3 | (reserved) | UINT8 | — | 0x00 |
| 4–7 | remaining_wh | FLOAT32 LE | Wh | SOC_GetRemainingAh() × pack_voltage |

3.5  NMT Frame  (COB-ID 0x000)

NMT commands are 2-byte frames with no CAN ID function code. The BMS listens and transitions state accordingly. It does NOT act as NMT master.

| CS — Command Specifier | Target Node ID (0 = all) |
| --- | --- |

| CS (hex) | Command | BMS Action |
| --- | --- | --- |
| 0x01 | Start | → OPERATIONAL — enables TPDO broadcast |
| 0x02 | Stop | → STOPPED — halts TPDO, heartbeat continues |
| 0x80 | Pre-Operational | → PRE-OPERATIONAL — TPDO paused, SDO still active |
| 0x81 | Reset Node | → INITIALISING — restarts NMT boot sequence |
| 0x82 | Reset Comm | → INITIALISING — restarts communication objects only |

3.6  Heartbeat Frame  (COB-ID 0x700 + NodeID)

NMT State (1 byte)

| Value (hex) | NMT State | TPDO Active |
| --- | --- | --- |
| 0x00 | INITIALISING | No |
| 0x7F | PRE-OPERATIONAL | No |
| 0x05 | OPERATIONAL | Yes — TPDO1–4 broadcast at 1 Hz |
| 0x04 | STOPPED | No |

3.7  SDO Server  —  Expedited Downloads (setpoints)

The BMS SDO server accepts expedited (≤4 byte) downloads to writable OD indices. Master sends to COB-ID 0x600+NodeID; BMS responds on 0x580+NodeID.

SDO Request (Master → BMS)  COB-ID 0x600+NodeID:

| CS | Index Lo | Index Hi | SubIdx | Data[0] | Data[1] | Data[2] | Data[3] |
| --- | --- | --- | --- | --- | --- | --- | --- |

CS for expedited 4-byte download = 0x23; 2-byte = 0x2B; 1-byte = 0x2F.

SDO Response (BMS → Master)  COB-ID 0x580+NodeID:

| 0x60 (ACK) | Index Lo | Index Hi | SubIdx | 0x00 | 0x00 | 0x00 | 0x00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

3.8  Writable Object Dictionary Entries

| Index | Sub | Type | Signal | Unit | Default | Firmware Action on Write |
| --- | --- | --- | --- | --- | --- | --- |
| 0x2000 | 0 | FLOAT32 | Pack Voltage | V | — | Read-only live value |
| 0x2001 | 0 | FLOAT32 | Pack Current | A | — | Read-only live value |
| 0x2002 | 0 | FLOAT32 | SOC % | % | — | Read-only live value |
| 0x2003 | 0 | FLOAT32 | SOH % | % | — | Read-only live value |
| 0x2004 | 0 | FLOAT32 | Temp NTC1 | °C | — | Read-only live value |
| 0x2005 | 0 | FLOAT32 | Temp NTC2 | °C | — | Read-only live value |
| 0x2006 | 0 | UINT16 | Fault Mask | — | — | Read-only live value |
| 0x2007 | 0 | UINT8 | ATC Status | — | — | Read-only live value |
| 0x200C | 1–6 | FLOAT32 | Cell Voltages 1–6 | V | — | Read-only live values |
| 0x3000 | 0 | FLOAT32 | Max Discharge Current | A | 60.0 | Fault_SetThresholds() immediate |
| 0x3001 | 0 | FLOAT32 | ATC Start Temp | K | 278.15 | ATC_SetStartTemp() immediate |
| 0x3002 | 0 | FLOAT32 | ATC Stop Temp | K | 288.15 | ATC_SetStopTemp() immediate |
| 0x3003 | 0 | UINT8 | Save to Flash | — | — | Write 0x01 → SaveSystemStateToFlash() |

4.  CAN 2.0B Extended Raw  (J1939-style)

Raw CAN 2.0B Extended mode transmits BMS telemetry as 6 periodic frames using 29-bit extended IDs in the J1939 PGN address space. No application-layer protocol overhead. Suitable for generic CAN analysers, custom ground station software, and non-UAV systems that cannot run DroneCAN or CANopen.

4.1  Physical Layer

| Parameter | Value |
| --- | --- |
| Peripheral | FDCAN1 (FDCAN_CLASSIC_CAN, FDCAN_EXTENDED_ID) |
| ID Format | 29-bit Extended — upper byte = 0x18 (J1939 PDU format) |
| Bitrate | GUI-selectable: 250 kbit/s / 500 kbit/s / 1 Mbit/s |
| Node ID | settings.can_node (1–127), appended to base CAN ID |
| TX Rate | 6 frames × 1 Hz (telemetry burst) |
| RX | Promiscuous receive (any frame ACK'd) — used by AutoSelect probe |

4.2  CAN ID Construction

| 0x18 [28:24] | PGN Hi [23:16] | PGN Lo [15:8] | Node ID [7:0] |
| --- | --- | --- | --- |

Base PGN for BMS telemetry: 0xFF50xx. Full base address: 0x18FF5000. Node ID is OR'd into the lowest byte, allowing up to 254 BMS nodes on one bus.

4.3  Frame Definitions

Frame 0: Pack Summary  —  0x18FF5000 + NodeID

| Byte(s) | Signal | Type | Unit | Source |
| --- | --- | --- | --- | --- |
| 0–3 | pack_voltage_v | FLOAT32 LE | V | SOC_GetPackVoltage() |
| 4–7 | pack_current_a | FLOAT32 LE | A | SOC_GetCurrent() (negative = discharge) |

Frame 1: State of Charge / Health  —  0x18FF5100 + NodeID

| Byte(s) | Signal | Type | Unit | Source |
| --- | --- | --- | --- | --- |
| 0–3 | soc_percent | FLOAT32 LE | % | SOC_GetPercent() |
| 4–7 | soh_percent | FLOAT32 LE | % | SOH_GetLifeData().soh_percent |

Frame 2: Temperature  —  0x18FF5200 + NodeID

| Byte(s) | Signal | Type | Unit | Source |
| --- | --- | --- | --- | --- |
| 0–3 | temp_ntc1_celsius | FLOAT32 LE | °C | temp_celsius |
| 4–7 | temp_ntc2_celsius | FLOAT32 LE | °C | temp2_celsius |

Frame 3: Faults & Capacity  —  0x18FF5300 + NodeID

| Byte(s) | Signal | Type | Unit | Source |
| --- | --- | --- | --- | --- |
| 0–1 | fault_mask | UINT16 LE | — | Fault_GetBitmask() — see §5 for bit definitions |
| 2–5 | remaining_ah | FLOAT32 LE | Ah | SOC_GetRemainingAh() |
| 6 | atc_status | UINT8 | — | ATC_GetStatus() 0=off 1=heating |
| 7 | (reserved) | UINT8 | — | 0x00 |

Frame 4: Cell Voltages 1–4  —  0x18FF5400 + NodeID

| Byte(s) | Signal | Type | Unit | Source |
| --- | --- | --- | --- | --- |
| 0–1 | cell_1_mv | UINT16 LE | mV | hDaly.data.cellVmV[0] |
| 2–3 | cell_2_mv | UINT16 LE | mV | hDaly.data.cellVmV[1] |
| 4–5 | cell_3_mv | UINT16 LE | mV | hDaly.data.cellVmV[2] |
| 6–7 | cell_4_mv | UINT16 LE | mV | hDaly.data.cellVmV[3] |

Frame 5: Cell Voltages 5–8  —  0x18FF5500 + NodeID

| Byte(s) | Signal | Type | Unit | Source |
| --- | --- | --- | --- | --- |
| 0–1 | cell_5_mv | UINT16 LE | mV | hDaly.data.cellVmV[4] |
| 2–3 | cell_6_mv | UINT16 LE | mV | hDaly.data.cellVmV[5] |
| 4–5 | cell_7_mv | UINT16 LE | mV | hDaly.data.cellVmV[6] (0 if <7 cells) |
| 6–7 | cell_8_mv | UINT16 LE | mV | hDaly.data.cellVmV[7] (0 if <8 cells) |

5.  AutoSelect Protocol  (FSM)

AutoSelect is a boot-time finite-state machine that probes for a DroneCAN flight controller. If detected, it locks to DroneCAN. If not, it falls back to CAN 2.0B Extended. The selection is not persisted to flash automatically (to prevent wear); it can be saved via GUI or SDO 0x3003.

5.1  State Machine

| State | Duration | Active Stack | Detection Condition | On Timeout / Fail |
| --- | --- | --- | --- | --- |
| PROBE_DRONECAN | 5 000 ms | libcanard + FDCAN1 @ 1Mbit/s | UAVCAN NodeStatus DTID=341 received from any node OR DNA alloc response received | → PROBE_CAN20B |
| PROBE_CAN20B | 3 000 ms | Raw FDCAN1 CAN 2.0B Ext | Any frame received in FIFO0 (HAL_FDCAN_GetRxMessage returns HAL_OK) | → LOCKED_CAN20B (safe fallback) |
| LOCKED_DRONECAN | ∞ | libcanard | FC heartbeat detected in probe | No transition |
| LOCKED_CAN20B | ∞ | Raw CAN 2.0B Ext | Bus activity detected OR probe timed out | No transition |

5.2  AutoSelect Detection Frames

During PROBE_DRONECAN phase, the BMS parses incoming frames looking for UAVCAN NodeStatus (DTID 341). The detection extracts data-type ID from bits [23:8] of the 29-bit extended CAN ID.

| CAN ID Bits | Field | Value Tested | On Match |
| --- | --- | --- | --- |
| [28:29] — 29-bit ExtID | ID Type | FDCAN_EXTENDED_ID | Continue parsing |
| [23:8] — 16-bit slice | Data Type ID | 0x0155 (341 decimal) | fc_detected = true → LOCKED_DRONECAN |
| [7:0] — 8-bit slice | Source Node ID | Any (0 = anonymous allowed) | Logged (not filtered) |

5.3  CLI Status Response  (CMD 0x61)

While probing, the GUI polls CMD 0x61 every 500 ms to display real-time status. The 7-byte response encodes the current protocol settings and AutoSelect FSM state.

| Proto [0] | Baud Lo [1] | Baud [2] | Baud [3] | Baud Hi [4] | CO NodeID [5] | AutoState [6] |
| --- | --- | --- | --- | --- | --- | --- |

| Byte | Field | Values |
| --- | --- | --- |
| 0 | can_protocol | 0=DroneCAN 1=CANopen 2=CAN2.0B 3=AutoSelect |
| 1–4 | can_bitrate_hz | UINT32 LE: 250000 / 500000 / 1000000 |
| 5 | canopen_node_id | 1–127 |
| 6 | auto_state | 0=Probing DroneCAN 1=Probing CAN2.0B 2=Locked DroneCAN 3=Locked CAN2.0B 4=N/A |

6.  Fault Status Bitmask  (Used across all protocols)

A 13-bit fault word is broadcast in every protocol. Bits are set by Fault_Update() on each firmware cycle and cleared when the condition resolves. The same bitmask appears in DroneCAN BatteryInfo.status_flags (byte 24), CANopen TPDO4 bytes 0–1, CAN 2.0B Frame 3 bytes 0–1, and CLI 0x61 responses.

| Bit | Mask (hex) | Fault Name | Trigger Condition |
| --- | --- | --- | --- |
| 0 | 0x0001 | overVoltageError | Pack voltage > OV threshold |
| 1 | 0x0002 | underVoltageError | Pack voltage < UV threshold |
| 2 | 0x0004 | overCurrentError | Discharge current > OC threshold |
| 3 | 0x0008 | chargeOverCurrentError | Charge current > charge OC threshold |
| 4 | 0x0010 | cellOverVoltageError | Any cell > cellMax_V threshold |
| 5 | 0x0020 | cellUnderVoltageError | Any cell < cellMin_V threshold |
| 6 | 0x0040 | cellDiffError | Max−min cell voltage > cellDiff_mV |
| 7 | 0x0080 | overTemperatureError | NTC1 > OT threshold |
| 8 | 0x0100 | overTemperature2Error | NTC2 > OT threshold |
| 9 | 0x0200 | underTemperatureError | NTC1 < UT threshold |
| 10 | 0x0400 | underTemperature2Error | NTC2 < UT threshold |
| 11 | 0x0800 | thermistorNotConnectedError | NTC1 ADC near rail (open circuit) |
| 12 | 0x1000 | thermistor2NotConnectedError | NTC2 ADC near rail (open circuit) |

7.  CLI Protocol Control Commands  (USART2 @ 115200 8N1)

The firmware CLI frame protocol (cli_frame.c) encodes commands as: [0xAA] [LEN] [CMD] [PAYLOAD...] [CHECKSUM]. Two commands were added for CAN protocol control.

7.1  CMD 0x60 — SET_CAN_PROTOCOL

| 0xAA (SOF) | 0x06 (LEN) | 0x60 (CMD) | proto [u8] | baud Lo | baud | baud | baud Hi | co_id [u8] | CKSUM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Field | Bytes | Values |
| --- | --- | --- |
| proto | 1 (u8) | 0=DroneCAN 1=CANopen 2=CAN2.0B Extended 3=AutoSelect |
| baud | 4 (u32 LE) | 250000 / 500000 / 1000000 (0 = keep current / default 1M) |
| co_id | 1 (u8) | CANopen node ID 1–127 (ignored for non-CANopen protocols) |

7.2  CMD 0x61 — GET_CAN_PROTO_STATUS  (poll response)

Send with 0-byte payload. Firmware responds immediately with a 7-byte payload (see §5.3 for byte layout).

| 0xAA (SOF) | 0x00 (LEN) | 0x61 (CMD) | CKSUM |
| --- | --- | --- | --- |

8.  Bitrate Configuration  (FDCAN1, PCLK1 = 170 MHz)

FDCAN1 is clocked from PCLK1 = 170 MHz (FDCAN_CLOCK_DIV1). Bitrate is set by CanProto_SetBitrate() which looks up pre-validated prescaler/timing values from a fixed table.

| Bitrate | Prescaler | Seg1 (TQ) | Seg2 (TQ) | SJW | TQ Period | Bit Time (ns) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 Mbit/s (default) | 34 | 3 | 1 | 1 | 200 ns | 200 × (1+3+1) = 1000 ns ✓ |
| 500 kbit/s | 34 | 7 | 2 | 1 | 200 ns | 200 × (1+7+2) = 2000 ns ✓ |
| 250 kbit/s | 34 | 15 | 4 | 1 | 200 ns | 200 × (1+15+4) = 4000 ns ✓ |

Document Revision History

| Rev | Date | Author | Change Summary |
| --- | --- | --- | --- |
| 1.0 | 2026-06-27 | DFI India / Claude AI | Initial release — all 3 CAN protocols + AutoSelect + Fault Bitmask + CLI commands |