Eionic 12S/14S Master BMS — Firmware Developer Guide

- Target: STM32G474RET6 (Cortex-M4F, 512 KB flash, 128 KB SRAM) Modular Development

- Project: Eionic 12S/14S Master BMS (CubeMX project STM32G474RET6_SOC)

- Build system: CMake + arm-none-eabi-gcc (STM32CubeMX-generated skeleton)

- Audience: engineers picking up this firmware for the first time

1. What this firmware is

The Eionic Master BMS is a supervisory / telemetry battery-management controller. It does not do cell balancing or contactor switching itself — a Daly BMS slave board handles cell measurement and balancing. This firmware sits above it and is responsible for:

| Responsibility | Module |
| --- | --- |
| High-accuracy pack current measurement | current_sensor.c (ACS37612 → ADC1) |
| Cell/pack voltage acquisition | daly_bms.c (UART5, Daly protocol) |
| Pack temperature | NTC10K.c (2× 10 kΩ NTC → ADC2) |
| State of Charge fusion | soc.c + lc_estimator.c |
| State of Health / life counters | soh_estimator.c |
| Fault detection & latching | fault_detector.c |
| Charger control (Solterra, CC/CV) | charger_config.c |
| Telemetry over DroneCAN/UAVCAN | Droncan/ + bmsNode.c |
| Telemetry to vehicle display | can_frame_handlers.c |
| Host/GUI command protocol | cli_frame.c, can_cli_bridge.c, can_gui_cmds.c |
| BLE telemetry bridge | inline in main.c (UART2) |
| Persistent calibration + config | calib_store.c → w25q64.c |
| Wear-levelled event log | error_log_store.c → w25q64.c |
| Low-power / deep sleep | power_manager.c, power_manager_port.c |
| Status indication | led_status.c |

Everything is bare-metal. There is no RTOS. Execution is a single cooperative super-loop in main() driven by a 100 ms timer flag, plus a small number of ISRs that only ever stage data for the loop to process.

2. Repository layout

STM32G474RET6_SOC/
├── Core/
│   ├── Inc/            application + driver headers
│   └── Src/            application + driver sources
├── Droncan/            libcanard + generated DSDL message (de)serialisers
│   ├── inc/            canard.h + uavcan.*.h / dronecan.*.h
│   └── src/            canard.c, bmsNode.c, uavcan.*.c
├── Drivers/            ST HAL + CMSIS (vendor, do not edit)
├── GUI/
│   └── stm32_cli_tool_v6.py    host-side Tk GUI / CLI tool
├── cmake/              toolchain files + CubeMX subproject
├── CMakeLists.txt      *** the authoritative source list ***
├── STM32G474RET6_SOC.ioc   CubeMX project (regenerating overwrites MX_* funcs)
├── STM32G474XX_FLASH.ld    linker script
└── SOC_SYSTEM_DOCUMENTATION.md   deep-dive on the SOC algorithm only

2.1 Files present but not compiled — read this first

The following .c files exist in Core/Src but are absent from CMakeLists.txt, so they are not in the binary. Do not assume they are live code:

- soc_extended.c

- fault_confidence.c

- signal_filter.c

- pm_recovery.c

- daly_bms_new.c

- soh_test.c

- main.c.bak (stale backup)

Related trap: main.c declares the globals hFilter (SignalFilter_Handle_t) and hFaultConf (FaultConfidence_Handle_t) and includes their headers, but never calls a single function from either module. They are dead symbols that survive linking only because nothing references them. If you “fix” a bug in signal_filter.c, nothing will change on hardware.

There are also several archive zips in the project root (Core.zip, Core_storageMode.zip, Core_Charger_working.zip, Core_BLE_Compressed.zip) — historical snapshots, not build inputs.

Rule of thumb: CMakeLists.txt is the single source of truth for what ships. Check it before touching anything.

3. Hardware interfaces

3.1 Clock tree

SystemClock_Config() — HSI16 → PLL (M=4, N=85, R=2) → SYSCLK 170 MHz, AHB/APB1/APB2 = /1, FLASH_LATENCY_4, voltage scaling Range 1 Boost. LSI is enabled and feeds the IWDG and the RTC (the RTC is what wakes the MCU from Stop mode).

3.2 Peripheral map

| Peripheral | Use |
| --- | --- |
| ADC1 ch1 | ACS37612 current sensor, TIM1-triggered (~2 kHz), interrupt-driven |
| ADC2 ch12 / ch14 | NTC1 / NTC2 thermistors, TIM1 TRGO triggered, rank 1 & 2 |
| TIM1 | ADC trigger source (TRGO) |
| TIM5 | 100 ms system tick → sets tim5_flag, the super-loop heartbeat |
| TIM6 | auxiliary periodic tick |
| TIM15 | 100 ms LED status tick |
| UART5 @ 9600 8N1 | Daly BMS slave link |
| USART2 @ 115200 | BLE module bridge (TX telemetry); legacy CLI RX is drained + discarded |
| FDCAN1 | Classic CAN 2.0B — DroneCAN, Solterra charger, display, GUI/CLI tunnel |
| QUADSPI1 | W25Q64 8 MB external flash |
| RTC (LSI) | wall-clock timestamps + wake timer for Stop mode |
| IWDG | ~4 s watchdog, refreshed once per super-loop iteration |

3.3 GPIO (from Core/Inc/main.h)

| Signal | Pin |
| --- | --- |
| LED1_D8 | PA1 |
| LED2_D9 | PC3 |
| LED3_D10 | PC2 |
| LED4_D11 | PC1 |
| BLUE_LED | PC8 |
| GREEN_LED | PA9 |
| RED_LED | PA10 |
| STB_CAN (transceiver standby) | PC9 |
| CAN_EN (transceiver enable) | PA8 |
| wakeButton | PB9, EXTI9_5, rising edge, pull-up |

4. Execution model

4.1 Boot sequence (main())

HAL_Init()
SystemClock_Config()                       170 MHz
MX_*_Init()                                GPIO, ADC1/2, TIM1/5/6/15, UART5, USART2,
                                           QSPI, RTC, FDCAN1, IWDG
CurrentSensor_Init()
CalibStore_Init() / CalibStore_Load()      QSPI flash, magic 0xCA1B000D
ErrorLog_Init()
  └── log RCC reset flags (IWDG/WWDG/BOR/PIN/SFT/LPWR/OBL) then clear them
if (calValid):  restore zero-offset, coupling factor, NTC B-values,
                DroneCAN settings, charger profile, device identity, SOC config
else:           CurrentSensor_StartZeroCalibration() + all defaults
Charger_RuntimeInit()                      never persisted — always re-derives live
load_settings() / canardInit() / canardSetLocalNodeID()
FDCAN1_ConfigFiltersAndStart()             *after* canard exists, so RX has a target
TIM5, TIM15 start; LedStatus_Init()
SOH_Init(), DalyBMS_Init() + StartUpdate()
SOC_Init(persisted soc_percent)            no 50% default reset
LC_InitWithConfig()
CLI_Frame_Init(&huart2)
Fault_Init(validated thresholds or defaults)
ADC2 self-calibrate, ADC2 start IT, TIM1/TIM6 start
PM_Init()                                  must be after RTC + IWDG init
→ while(1)

The boot-time reset-flag logging is deliberate and load-bearing: it is the only way to distinguish a firmware hang (IWDG flag) from a brown-out under high charge current (BOR flag) after the fact. Note BOR also sets on a normal cold power-up on this series.

4.2 The super-loop

Runs free (no delay). Every iteration:

- HAL_IWDG_Refresh()

- Charger_ProcessStatusFrame() — drains the charger RX staging buffer and runs the 5 s comm-timeout watchdog. Cheap and O(1) when idle.

- FDCAN1_HealthCheck() on a 500 ms throttle — fast bus-error/EMI recovery.

Then, gated on tim5_flag (100 ms):

| Step | Work |
| --- | --- |
| — | DroneCAN DNA request if node ID unassigned; send_BatteryInfo() |
| 1 s block | process1HzTasks(), CAN health check + log, Solterra charger command TX |
| — | drain CAN RX queue (deferred from ISR); drain libcanard TX queue |
| 1 | current-sensor calibration flags |
| 1b/1c | consume NTC1 / NTC2 averaged samples |
| 2 | DalyBMS_Task() state machine |
| 3 | read current (mA → A) |
| 3b | update fault detector with a balancer-safe voltage snapshot |
| ~200 ms | display telemetry: BMS_VIT, BMS_OvrView, Cell_DeltaV, BMS_Capacity, BMS_CellVolts |
| 3c | LED error state |
| 5 | SOC coulomb counting + voltage correction |
| 6 | hard calibration anchors (0 % at UV, 100 % at full) |
| 6b | auto-save to flash if SOC % changed (throttled to ≥5 min) |
| 7 | periodic CLI telemetry frame (100 ms) |
| — | BLE telemetry on an independent 1 s cadence |
| 8 | UART2 legacy CLI RX drained and discarded |
| 8a | CAN-CLI tunnel commands |
| 8c | legacy fixed-format GUI/BUSMASTER CAN commands |
| 8b | host-command loss watchdog |
| 9 | PM_Task() — must be last in the loop |

4.3 Interrupt policy

ISRs are deliberately thin. The pattern throughout is latch in ISR, decode in loop:

- HAL_FDCAN_RxFifo0Callback() pushes into an 8-deep can_rx_queue[] ring, or calls Charger_LatchRawFrame() which only memcpy’s raw bytes. No decode, no state machine at IRQ priority — a flooding charger cannot add unbounded ISR work.

- ADC1_2_IRQHandler runs the per-sample current Kalman filter and NTC accumulation.

- USART2_IRQHandler uses direct RXNEIE-driven reception (not HAL single-byte Receive_IT) to avoid a silent chain-break failure mode.

- EXTI9_5_IRQHandler — wake button.

- RTC_WKUP_IRQHandler — sleep wake timer.

Full ISR list lives in Core/Src/stm32g4xx_it.c.

5. Measurement chain

5.1 Current — current_sensor.c

ACS37612 Hall sensor → ADC1 ch1, TIM1-triggered. Per-sample 1-D Kalman filter (Q = 0.001, R = 4), then Q16 fixed-point, then mV, then (mV − zeroOffset) × couplingFactor = mA.

Calibration states: CS_STATE_ZERO_CAL (1000 samples at zero current) → CS_STATE_KNOWN_CAL (operator supplies a known current) → CS_STATE_RUNNING. Both results persist to QSPI flash.

5.2 Cells and pack voltage — daly_bms.c

UART5, 13-byte frames [0xA5][0x40][CMD][0x08][DATA×8][CHK], non-blocking. Polled command sequence: 0x90 (pack V/I/SOC), 0x91 (min/max cell + index), 0x95 (all cell voltages, multi-frame).

The Daly’s active balancer injects switching glitches, so each cell goes through a 100-sample rolling median plus a 2-state EKF ([V_cell, V_rc_ripple], ripple τ ≈ 10 s). Pack voltage gets a simple 1-D KF (Q = 0.01, R = 0.5). lastKnownGoodNCells guards the multi-frame 0x95 parse against a miscount while the balancer is switching.

5.3 Temperature — NTC10K.c

Two 10 kΩ NTCs on ADC2 ch12 / ch14, TIM1-triggered, averaged in the ISR and consumed by the loop. B-values (g_calib_b, g_calib2_b) are calibratable and persisted. Sentinel values matter: -999.0f = not yet ready, 999.0f (CHARGER_TEMP_SENSOR_FAIL_C) = sensor invalid/missing.

5.4 SOC / SOH

Coulomb counting is primary; OCV lookup is a secondary, rest-gated correction; a Kalman filter fuses them; hard anchors force 0 % at UV and 100 % at detected full charge. lc_estimator.c adds a Wh/range-style “load capacity” estimate initialised from the real pack config, not hardcoded defaults. soh_estimator.c tracks cycle count and internal resistance growth.

See SOC_SYSTEM_DOCUMENTATION.md for the algorithm-level detail — it is accurate and still current.

6. Fault handling — fault_detector.c

Faults are a latched 32-bit bitmask retrieved with Fault_GetBitmask().

| Bit | Fault | Default threshold |
| --- | --- | --- |
| 0 | FAULT_BIT_OV pack overvoltage | 58.8 V |
| 1 | FAULT_BIT_UV pack undervoltage | 42.0 V |
| 2 | FAULT_BIT_OC discharge overcurrent | 300 A |
| 3 | FAULT_BIT_CELL_DIFF imbalance | 15000 mV (effectively disabled) |
| 4 | FAULT_BIT_OT overtemperature | 55 °C |
| 5 | FAULT_BIT_UT undertemperature | −10000 °C (disabled) |
| 6 | FAULT_BIT_THERM_MISSING | — |
| 7 | FAULT_BIT_OT2 | 55 °C |
| 8 | FAULT_BIT_CELL_OV per-cell OV | 4.26 V |
| 9 | FAULT_BIT_CELL_UV per-cell UV | 3.00 V |
| 10 | FAULT_BIT_UT2 | — |
| 11 | FAULT_BIT_THERM2_MISSING | — |
| 12 | FAULT_BIT_CHARGE_OC charge overcurrent | 150 A |
| 13 | FAULT_BIT_CURRENT_SENSOR_FAULT | — |

Bit 13 is not an over-current condition — it means the sensor reading itself is untrustworthy (NaN/Inf/implausible). Do not conflate it with bits 2 or 12.

Thresholds restored from flash are validated field-by-field via ThrOrDefault() (each falls back to its own FAULT_*_DEFAULT if out of the accept range) and then as a consistent set via ValidateFaultThresholds() — if e.g. uv >= ov, the entire default set is taken and a log entry is written. This exists because a partially-written or older-layout record used to be able to install a nonsense threshold that presented as a phantom fault.

Fault_WarnIfOvBelowCvTarget() catches the configuration mistake of an OV threshold below the charger’s CV target — which would trip a fault every time charging completes.

7. Communications

7.1 FDCAN1 — one bus, four protocols

Classic CAN 2.0B. Five extended-ID hardware acceptance filter banks (StdFiltersNbr = 0 — there are no 11-bit filter banks, so every ID this firmware accepts must be extended):

| Filter idx | Purpose |
| --- | --- |
| 0 | Charger exact match |
| 1 | Display range |
| 2 | DroneCAN catch-all |
| 3 | GUI fixed-format commands |
| 4 | Storage-mode command |

ID map:

| ID | Direction | Meaning |
| --- | --- | --- |
| 0x1806E5F4 | BMS → Charger | Solterra charge command, 1 Hz |
| 0x18FF50E5 | Charger → BMS | Solterra status broadcast (DLC ≥ 5) |
| 0x1FF820 | BMS → Display | BMS_VIT |
| 0x1FF810 | BMS → Display | BMS_OvrView |
| 0x1FF710 | BMS → Display | Cell_DeltaV |
| 0x1FF730 | BMS → Display | BMS_CellVolts |
| 0x1FF740 | BMS → Display | BMS_Capacity (remaining Ah) |
| 0x1FF821 | Display → BMS | Storage_Mode_Cmd, DLC 1, data[0]=0x01 = ON |
| 0x1FF900–0x1FF906 | PC → BMS | legacy fixed-format GUI commands |
| 0x1FF907 | PC → BMS | CAN-CLI tunnel request |
| 0x1FF908 | BMS → PC | CAN-CLI tunnel response |

The storage-mode ID being extended was confirmed by BUSMASTER capture — worth knowing, because assuming it was standard would require repartitioning the FDCAN message RAM.

7.2 DroneCAN / UAVCAN — Droncan/

libcanard with a 4 KB memory_pool. bmsNode.c implements:

- NodeStatus heartbeat (1 Hz)

- uavcan.equipment.power.BatteryInfo (rate set by the TELEM_RATE parameter, default 10)

- GetNodeInfo, RestartNode, param.GetSet, param.ExecuteOpcode

- Dynamic Node Allocation (DNA) when no node ID is configured

Three persisted parameters: CAN_NODE (default 5), BATT_INDEX, TELEM_RATE. Note that handle_param_GetSet() does not enforce its own advertised 0–127 range on write, which is why main() clamps defensively before calling DeviceIdentity_SyncNodeId().

7.3 Host command protocol — cli_frame.c

A framed binary protocol carried over either USART2 or the CAN tunnel (0x1FF907/0x1FF908), with identical command IDs on both transports. Selected commands:

Telemetry (BMS → host)

| ID | Payload |
| --- | --- |
| 0x01 SOC_DATA | SOC %, Ah, I, V (16 B) |
| 0x02 BMS_DATA | summary (32 B) |
| 0x03/0x07 temp / temp2 |  |
| 0x04 FAULT_STATUS |  |
| 0x05 RTC_DATA |  |
| 0x06 ERROR_LOG_DATA |  |
| 0x08 CELL_EXTREMA | min idx/V, max idx/V (10 B) |
| 0x0A BMS_LIFE | cycle/life counters (16 B) |
| 0x0C CELL_VOLT_ALL | nCells + n×float32 |
| 0x50 LC_DATA | load-capacity estimate |
| 0x56 PM_STATUS | power-manager snapshot |
| 0x62 DEVICE_IDS_DATA | identity (52 B) |
| 0x76 BLE_LIVE | packed 15 B BLE telemetry |
| 0x77 BLE_BATTERY_INFO | serial + mfg date (19 B) |

Commands (host → BMS)

| ID | Action |
| --- | --- |
| 0x10 / 0x11 | zero calibration / known-current calibration |
| 0x12 / 0x13 | force SOC 100 % / 0 % |
| 0x14 | save calibration to flash |
| 0x15 / 0x16 | NTC1 / NTC2 temperature calibration |
| 0x20 / 0x21 | set fault thresholds / set SOC config (100 B) |
| 0x30 / 0x31 / 0x32 | set RTC / read error logs / clear error logs |
| 0x51–0x53 | LC zero, set SOH, reset life counters |
| 0x54 / 0x55 | clear latched faults / reset drive state |
| 0x57 / 0x58 | set / get power-manager config |
| 0x60 / 0x61 | set / get device identity |
| 0x63 / 0x64 | set / get charger override |
| 0x66 | CAN ping (echo) |

The 0x57 PM_Config_t wire layout is fixed at 12 bytes little-endian, matching the Python GUI’s struct.pack("<HHHHBBBB"), and is packed field-by-field on the firmware side so compiler padding can never desynchronise the two. Preserve this if you add fields — append only.

7.4 BLE bridge

USART2 at 115200 to a BLE module, 1 s cadence, independent of the 100 ms CLI cadence. Payloads use scaled integers (0x76, 0x77) to fit BLE MTUs — e.g. pack voltage in centivolts, current in deciamps signed, temperature as °C + 40, cell voltage × 20.

8. Charger control — charger_config.c

Solterra charger over CAN, commanded at 1 Hz on 0x1806E5F4. Frame is voltage ×10 and current ×10, big-endian, plus a control byte where 0x00 = enabled/START and 0x01 = disabled/STOP (confirmed against real hardware — the polarity is counter-intuitive).

Runtime state machine (hChgRt, never persisted, re-derives within seconds of boot): DISCONNECTED → READY → CHARGING, plus FAULT, INHIBITED, TIMEOUT (5 s silence).

Control flow, in order:

- Charger_ComputeCommand() runs CC → CV → taper-termination, factoring in SOC, fault bitmask, gate temperature and storage-mode request.

  - CC: command chargeCurrentFull_A until the pack voltage target is reached

  - CV: charger self-limits, current tapers

  - when charger-reported current ≤ cvCutoffCurrent_A → charge_done = 1, enabled = 0

- Hysteresis: once charge_done, stay stopped until SOC drops to ≤ 98 %. This prevents on/off chatter at 99–100 %. Note that hitting SOC 100 % deliberately does not force a hard stop — letting CV finish is what actually fills the pack.

- Thermal gate: hard block at ≥ 50 °C, resume at ≤ 47 °C with a 0.8 °C tolerance band. It is a sticky latch (rt->tempStartBlocked), and every transition is logged with the causing temperature — without that, a thermally-blocked session looks like an unexplained silent STOP.

- Host override (0x63) applied last. It can only ever force STOP; there is no code path from this flag to a forced START, so it cannot bypass a safety trip. Clearing it merely stops overriding.

- Storage mode (CAN 0x1FF821): targets storageVoltageSetpoint_mV (default 3800 mV/cell) instead of the normal per-cell setpoint.

Status bits reported back: CV_PHASE (0x01), CHARGE_DONE (0x02), TEMP_BLOCKED (0x04), HOST_STOPPED (0x08), STORAGE_MODE (0x10).

9. Persistent storage — W25Q64 (8 MB QSPI)

flash_map.h splits the device into three ~2.66 MB partitions:

| Partition | Offset | Size | Contents |
| --- | --- | --- | --- |
| 1 | 0x000000 | 0x2AA000 | Calibration store |
| 2 | 0x2AA000 | 0x2AA000 | Error log ring |
| 3 | 0x554000 | remainder | Firmware / OTA (reserved) |

9.1 Calibration store — calib_store.c

One CalibStore_Data_t guarded by magic 0xCA1B000D. Holds: current-sensor zero offset and coupling factor, both NTC B-values, DroneCAN settings, SOC config + OCV curve, all fault thresholds, charger profile, device identity, and the last-known SOC %. Written on explicit 0x14 SAVE_CAL, and auto-saved when SOC % changes — throttled to at most once per 5 minutes to protect flash endurance.

The magic value was bumped when the struct grew past its previous 48-byte layout. Bump it again if you change the layout, otherwise an old record will be read as a valid new one. Note the restore path defends against this anyway (cellCount plausibility check, forced NUL-termination of identity strings), but the magic is the real guard.

9.2 Error log — error_log_store.c

An automotive-style wear-levelled ring:

- 20 × 4 KB sectors in partition 2, written round-robin

- each record carries magic 0xE10A, an RTC timestamp, a severity, a fault bitmask, and up to 58 chars of message (ERROR_LOG_MSG_MAX_LEN)

- severities: INFO (0), WARNING (1), CRITICAL (2)

- ErrorLog_Add() and ErrorLog_QueueDeferredEx() enqueue rather than block — QSPI erase must never stall the super-loop

- read back page-by-page via 0x31, erased via 0x32

ERRLOG_SECTOR_COUNT can be raised for more history; slot count and wear-levelling scale automatically.

10. Power management — power_manager.c / power_manager_port.c

Configuration lives entirely in power_manager_conf.h. Setting PM_ENABLE = 0 compiles the whole subsystem out; every entry point still links as a no-op, so no call sites need guards.

10.1 Modes

| Mode | Value | Default dwell before next mode |
| --- | --- | --- |
| PM_MODE_IDLE | 1 | 30 s → SLEEP |
| PM_MODE_SLEEP | 2 | 270 s → DEEP_SLEEP |
| PM_MODE_DEEP_SLEEP | 3 | 3600 s → PRO (0 = never) |
| PM_MODE_PRO_DEEP_SLEEP | 4 | 1800 s re-check (0 = button only) |

These values are wire-visible over CLI 0x56 — append, never renumber.

10.2 Wake sources

PM_WAKE_CAN (2), PM_WAKE_BUTTON (3, PB9), PM_WAKE_CURRENT (4, ≥ 3.0 A with 0.5 A hysteresis and 2 confirmations), PM_WAKE_FAULT (5), PM_WAKE_INTERVAL (6), PM_WAKE_RESET (7). PM_WAKE_TICK (1) is housekeeping, not a real wake.

10.3 Sleep inhibits and guards

Sleep is blocked while: cell delta > 50 mV, Daly data stale > 3 s, pack cycling, charger present, or CAN traffic seen within 10 s. On top of that:

- PM_MIN_SLEEP_DWELL_S (10 s) minimum dwell — prevents thrash

- wake-loop limiter: 6 wakes in 60 s trips a back-off

- CAN wake probation (300 ms–8 s, adaptive) and a spurious-wake limiter (10 → 60 s blackout), because a dominant-stuck bus would otherwise wake the MCU continuously

- fault indication window of 5 s with a 60-minute re-check interval

- IWDG period is chunked at 4002 ms across sleep so the watchdog never fires while stopped

PM_Task() is the last call in the super-loop by design — everything else must have finished its work for the tick before the MCU is allowed to consider stopping.

10.4 Known hardware limits

Sub-mA deep sleep is not achievable on the current board: the ACS37610 draws ~10 mA with no load switch, and the Daly slave has no rail switch. Reaching microamp figures requires board rework (load switches on both rails). PM_SENSE_EN / PM_VDIV_EN / PM_DALY_EN hooks already exist in power_manager_conf.h for when that rework lands.

11. Status indication — led_status.c

System states drive the RGB LED: SYS_STATE_IDLE (blue static), SYS_STATE_DRIVE and SYS_STATE_CHARGE (green static), SYS_STATE_FAULT (red static). Error codes LED_ERR_* map 1:1 onto the fault bits and are blinked out on the D8–D11 indicator LEDs. Driven by a TIM15 100 ms tick. The drive/charge/idle decision itself uses a dead-band timeout plus a spike-persistence counter (spike_persist_cnt, state_deadband_ticks) so a momentary current spike does not flip the displayed state.

12. Building

cmake -S . -B build -DCMAKE_TOOLCHAIN_FILE=cmake/gcc-arm-none-eabi.cmake \
      -DCMAKE_BUILD_TYPE=Debug
cmake --build build -j

Produces STM32G474RET6_SOC.elf. A prebuilt configuration also exists under config_default/. An alternative Clang toolchain file is provided at cmake/starm-clang.cmake.

When adding a source file, add it to target_sources() in CMakeLists.txt. This is not theoretical — the entire power-management subsystem was silently absent from the binary because power_manager.c and power_manager_port.c existed on disk but were never listed, and the same class of bug previously made can_gui_cmds.c unreachable.

Regenerating from the .ioc overwrites the MX_*_Init() functions and SystemClock_Config(). All application code lives inside /* USER CODE BEGIN */ … /* USER CODE END */ markers and is preserved — keep it that way.

13. Host tooling

GUI/stm32_cli_tool_v6.py is a Tk/ttkbootstrap application speaking the cli_frame protocol over serial. It provides live gauges, fault-threshold editing, an SOC/OCV curve editor, charger profile configuration, power-manager settings, error-log pagination, device identity, and CSV export. A PEAK-CAN variant carries the same command set over the CAN tunnel IDs.

When you change a protocol payload, the GUI’s struct format strings must change in lockstep. There is no version negotiation between the two.

14. Open issues and gotchas for newcomers

- Cell-count inconsistency — highest priority. Three places disagree:

  - main.c calls DalyBMS_SetCellCount(&hDaly, 16)

  - charger_config.h defaults to CHARGER_CONFIG_DEFAULT_CELLS = 12

  - power_manager_conf.h defines PM_CELL_COUNT = 14

- This value is load-bearing in at least four places: the fault derating table, the GUI’s L2 warning bands, the cell-voltage broadcast, and the CV charge target. Confirm the actual pack series count and make all three agree before trusting any charge or fault behaviour. The flash-restored charger_config.cellCount overrides its default at runtime if a valid record exists, which makes the disagreement easy to miss on a board that has been calibrated.

- Dead code that looks live — see §2.1. signal_filter.c, fault_confidence.c, soc_extended.c and friends are not compiled, and hFilter / hFaultConf are never used.

- A stale Core/Src/fdcan.h has previously shadowed the real header in Core/Inc. If FDCAN behaviour does not match the header you are reading, check which one the include path resolves.

- assert.h is included but <stdio.h> deliberately is not. String building uses strcpy/strcat plus the local AppendIntStr() helper, to avoid pulling several KB of newlib printf (and -u _printf_float) into flash. Do not introduce snprintf casually.

- CAN RX queue depth is 8. Deliberately small. If you add a high-rate accepted ID, check for overrun before increasing it.

- The legacy UART2 CLI RX path is drained and discarded, not dispatched. Commands must arrive via the CAN tunnel or the current framing. This surprises people debugging “why doesn’t my serial command work”.

- 0x00 means START in the Solterra control byte. Verified on hardware.

15. Suggested reading order for a new developer

- Core/Inc/main.h — pin map

- CMakeLists.txt — what actually builds

- main() init block, then the super-loop body — the whole control flow is there

- Core/Inc/fdcan.h — the CAN ID map, which explains most of the system’s external behaviour

- Core/Inc/cli_frame.h — the host protocol

- Core/Inc/charger_config.h then charger_config.c — the highest-consequence subsystem

- Core/Inc/power_manager_conf.h — every tunable in one place

- SOC_SYSTEM_DOCUMENTATION.md — the estimation maths